How to set a tagged VLAN

# For KS8842/KS8695P Switch WINCE version

To Enable the VLAN support for KS8842/KS8695P driver:

- Add compiler flag VLAN_ENABLE in the driver source file.

- Add BSP variable VLAN_ENABLE =1 to driver package.

Registry settings for VLAN support:

All settings use KS8842 as an example. For KS8695P all parameters are same. Only registry section key is ‘KS8695Pswitch’ and there are 5 LAN port settings for KS8695P.

[HKEY_LOCAL_MACHINE\Comm\KS88421\Parms\TcpIp]

"VlanId"=dword:7

Note: Set VlanId under TcpIp section will cause the driver insert a VLAN tag (VID will use ‘VlanId’ value).  To understand how the driver handles sending/receiving VLAN packets, please refer the ‘Receiving 802_1Q-Marked Packets.html’ and ‘Sending 802_1Q-Marked Packets.html’ come with the BSP/driver package.

[HKEY_LOCAL_MACHINE\Comm\KS8842Switch]

"Vlan"=dword:2

Note:  Set the Vlan=2 will enable tagged VLAN on switch hardware.  Set the Vlan=1 will enable port based VLAN on switch hardware (not explain here) . Set Vlan=0 will disable switch VLAN function.

[HKEY_LOCAL_MACHINE\Comm\KS8842Switch\VlanTable1]

"MemberShip"=dword:7

"FID"=dword:1

"VID"=dword:7

Note: To let the switch hardware forwarding a tagged VLAN packet, you must set a VlanTable and its VID equal the VLAN packets VID which you want to forward. If a VALN packet’s VID can not be found in any VlanTable, it will be dropped by hardware,

The KS8842/KS8695P switch supports 16 VLAN tables (VlanTable1-VlanTable16), so it can support 16 VLANs recognition. 1

For example: if networks connected by switch have 5 VLANs (assume VLAN ID are 1,2,3,4,5) and we only want switch to forward VLAN (1,3 and 5). We need to set 3 Vlan Tables, they VIDs are 1,3 and 5.

The VLAN Table parameters explanation:

"VID": The packet tagged by this VID, which you want to switch to forward.

"FID":  A mapping value for the VID, its value from 0x0-0xF, so 16 VLAN tables can      map to 16 FIDs. The Switch will use the FID (Not VLAN ID) and packet’s destination MAC address to lookup in Static MAC Tables and Dynamic MAC Tables  (The Dynamic MAC tables are a read only tables) to find ports the packet need to be forwarded.

"MemberShip":  If the Switch engine fails to find the packet’s destination MAC address in any Static MAC table and Dynamic MAC Table. The tagged packet will be broadcasted to ports based on this parameter.

For example: 	0x3 (bit pattern is 101) means broadcasting the tagged packet to port0 and port2. 0x7 (bit pattern is 111) means broadcasting the tagged packet to all ports (port0, port and port2).

[HKEY_LOCAL_MACHINE\Comm\KS8842Switch\StaticMACTable1]

"FID"=dword:1

"UseFID"=dword:1;

"ForwardingPorts"=dword:7

"Override"=dword:0;

"MAC"="12:34:56:AA:BB"

Note: To let the switch can directly send (not broadcast) a tagged VLAN packet to desired ports, we need to set a Static MAC Table.  The switch supports 16 StaticMacTable (StaticMacTabl1-StaticMacTabl6).

The Static MAC Table parameters explanation:

"FID":  (0x0-0xF) The value will be used for lookup by switch to see if it matches a FID defined in the any VLAN table and the packet's destination address matches the MAC address field in this table or not. If the two conditions are both satisfied, (which means the packet’s VID is found in a VLAN table and its destination MAC address is found in this Static MAC table), the packet will be directly sent to ports defined by "ForwardingPorts" parameter.

"UseFID": Normally it should be set to 1. If it is set to 0 (not use FID). The switch engine will only use packet’s destination MAC address to do a lookup. If a tagged packet's destination MAC address matches with this table's MAC parameter. The packet will be directly sent to ports defined by "ForwardingPorts" parameter.

Note: Setting the "UseFID" to 0, means KS8842 Switch ignores the VLAN ID and only using MAC address to decide forwarding.

"ForwardingPorts":  When forwarding conditions are satisfied, the tagged packet will be send to ports defined by this parameter.

Example: 	3 (bit pattern is 101) means send to port 0 and port 2.

7 (bit pattern 111) means send to all ports (port0, port1 and port2).

"Override":  If it is 1, it will force the Ports (specified by "ForwardingPorts" parameter) to enable transmit and receiving. (If they are original settings are disabled).

"MAC":  Destination MAC address which you want tagged packet to be sent to.

If a tagged packet’s destination MAC address matches this parameter, which means the ‘destination address match’ condition is satisfied.

[HKEY_LOCAL_MACHINE\Comm\KS8842Switch\Port0]

"MemberShip"=dword:7

"PVID"=dword:3

"Priority"=dword:3

"VLanFilteringEnable"=dword:0

"DiscardNonPVIDPk"=dword:0

"TagRemoval"=dword:0

"TagInsertion"=dword:0

"802.1pEnable"=dword:0

Note: Each port’s property on the switch can be set independently. But the

Host Port (in the KS8842 is port2 and in the KS8596P is port 5) settings should not be changed (all settings are zero except Membership is 7/0xF), because the Host Port behave fully controlled by driver software.  Any wrong settings for host port will cause un-expected results.

The port settings parameters explanation:

"MemberShip":  This parameter will be used only for Port Based VLAN. For Tagged VLAN, this parameter will be ignored.

"PVID":  Port VID. If a non-tagged or NULL tagged packet is received, this VID will be used for VLAN table look-up.

"Priority": If switch set to 802.1Q tagged Vlan mode, this parameter’s range is 0x00-0x07, which is ‘User Priority Field’ in the VLAN header.

If switch set to Port Based VLAN mode, this parameter’s range is 0x00-0x03.

"VlanFilteringEnable":  If it set to 1 (enabled), the switch will discard the packets whose VID port membership in VLAN table’s ‘MemberShip’ does not include the ingress port VID.

"DiscardNonPVIDPk": If it set to 1 (enabled), the switch discard packets whose VID does not match the ingress port default ID (PVID).

Note: If you want switch can forward more one VLAN, this parameter should be set to 0 (disabled).

"TagRemove": If it set to 1 (enabled), the switch removes 802.1Q/P tags from packets (If they are tagged packets). The switch will not modify packets, if the packets are without tags.

"TagInsetion": If it set to 1 (enabled), when the packets are output on the port, the switch adds 802.1Q tags to the packets which are non-tagged by using PVID parameter. The switch will not add tags to packets already tagged.

"802.1pEnable": If it set to 1, the switch will use 802.1p priority value.

Note: After you changed the VLAN registry settings in runtime. You should call OID_GEN_VLAN_ID NDIS call to let the driver update the new settings.