# BIOS Recovery Using the COM1 Flash Programmer

Revision: 1.01

Date: Sep 10, 2002

Diamond Systems Corporation

8430-D Central Avenue

Newark, CA. 94560

##### Scope

This document describes the procedure to recover the system BIOS when it has been lost due to a corrupted system EEPROM.

  - Overview

The recovery procedure requires another P.C. running Win95 or Win98. This host machine is connected to the target using a null modem serial cable connected to COM1 on the host and COM1 on the target. Three files are required on the host P.C: Zftrm.exe, Loadbios.com, and the file containing the BIOS, PRMBG.ROM. These files should be in the same directory.

  - Procedure

- Run Zftrm.exe on the host P.C. Enter the command:

“zftrm.exe loadbios.com PRMBG.ROM”

- Power up or reset the target while pressing the ZFIX key on the front panel, or power up the target with the A23 jumper installed.

If the software cannot recognize the flash device as an AMD or Atmel part then the device might have a hardware problem. If this is the case the software will exit with a message indicating that the device is not supported.

After the software recognizes the device, the entire flash chip is completely erased. This might take up to 90 seconds.

After the chip is erased the software reports “Flash Erased” and then starts downloading the BIOS image from the host and programming the chip. To completely download and reprogram the BIOS takes several minutes. While this happens characters are continuously scrolled across the screen of the host P.C. It is recommended that no other applications or tasks be running simultaneously.

When the device is completely reprogrammed with the BIOS the host will report the completion and the software exits.