# Prometheus BIOS 1.07M Onboard Flash Setup Instructions

Diamond Systems Corporation

8430-D Central Ave., Newark, CA 94560

(510) 456-7800, www.diamondsystems.com

Author: Steve Hopkins, Systems/Support Engineer.

### Preface

These instructions describe how to use the Prometheus onboard flash memory as a solid state flashdisk that can be used with MS-DOS and ROM-DOS. Prometheus contains a 2MB flash memory chip that is used to store the BIOS. The BIOS only occupies 256K bytes, leaving the rest available for use. The Prometheus BIOS version 1.07M provides support to convert the unused area into a disk drive. Once the Onboard Flash is set up this way, it can be accessed through a drive letter exactly like a floppy drive. It can also have OS system files transferred to make it bootable, so you don’t need any additional storage media.

The flash chip itself has a rated lifetime of 2,000,000 write cycles per sector. To extend the flash lifetime, the BIOS always uses the next available free location in the flash memory when writing files. Each time you write a file, even if it's to replace an existing file, a different memory location gets used. This way if you read / write the same file over and over, you don't wear out the same location in memory. This extends the life of the flash beyond 2,000,000 writes for the same file.

Please do not mistake this onboard flash for the removable Apacer Flashdisk Modules that we also sell with the Prometheus.

### Initial Setup

- Install BIOS version 1.07M or later if not already installed. Earlier versions of the Prometheus BIOS do not support this feature. Please see the BIOS recovery section of the users manual or the instructions included with the BIOS upgrade package on our website.

- Boot the system and enter the BIOS setup by pressing F2 early in the boot up.

- Go to Advanced->Advanced Chipset Control and enable the “Flash Virtual Drive”.

- Save and exit the BIOS by hitting F10. WARNING: Do not skip this step.

- The system will boot again. Reenter the BIOS after it boots by hitting the F2 key.

- Go to the Advanced->Advanced Chipset Control. Highlight “FLASH Initialize” then hit <enter>. You will see a percentage complete indicator. After completion the BIOS will return to the “Advanced” screen.

WARNING: When you press enter, the flash memory will be initialized and erased. Any data currently in the flash (except the BIOS) will be lost!

- Hit the <ESC> key to return to the main tabbed menus.

- Go to the Boot tab.

- Highlight “Removable Devices” and hit <enter>.

- The “Onboard Flash” will be listed after “Legacy Floppy Drives”. This will make it a non-bootable B: drive. You can change it to a bootable A: drive by hitting the <+> or <-> key to move it to the top. This should only be done after it has been formatted and loaded with operating system files, otherwise your system will not boot.

- Save and exit the BIOS by hitting the F10 key.

### OS Format

- Attach a floppy drive to Prometheus.

- Boot to a DOS disk in the Floppy drive.

- At the A: prompt type “format b: /s” to format the Onboard Flash drive with the system files.

- To make this drive a bootable A: drive, reboot the system and re-enter the BIOS. Go back to the Boot menu and change the boot order in the BIOS setup and list the “Onboard Flash” before the “Legacy Floppy Drive”.

### Known Limitations

- RFD (onboard flash drive) is not compatible with DOS expanded memory configuration of EMM386.EXE. NOEMS switch appended to the end of the EMM386 line in config.sys resolves this problem. Example config.sys, “device=c:\DOS\EMM386.exe NOEMS”

- The Onboard Flash has a limitation of 2,000,000 erase cycles so swap drives or virtual memory functions are not allowed.

- Currently we are shipping 2M byte Flash chips so Onboard Flash drives can be formatted with 1.45M bytes left over. 4 and 8M byte versions can be custom ordered. Contact sales@diamondsystems.com for more information.

- Only 16 bit OS’s are supported for this Onboard Flash. MSDOS 6.22 and ROMDOS 6.22 and 7.0 have been tested.  Other OS’s may work as long as it has some sort of 16-bit compatibility mode.

To receive a free copy of the latest BIOS along with upgrade instructions, contact tech support at support@diamondsystems.com or visit our website at www.diamondsystems.com .